# Baptistry - Preparing for a Baptism

The baptistry is comprised of three separate elements relating to baptisms:  The water pump, the waterfall, and the water heater.  It takes a few hours for the water to heat sufficiently for a baptism.  If the baptism is scheduled for a morning, it is advisable to pre-heat the water the day before the baptism.

Note:  The water heater will only heat water when the water pump is running.

- If the water pump is already running, and the heater is then turned on, the water will start being heated.

- If the water heater is already turned on, and the water pump then starts running, the water will start being heated.

- If the water pump and heater are both turned on, and either device is turned off, the water will stop being heated.

# Day Prior to a Baptism

- Turn the Water Pump and Waterfall On by activating the 8 HR mode on the On Demand Timer Switch.  This will run both the Water Pump and the Waterfall for 8 hours, which should be enough time to heat the pool.

- Turn on the Water Heater located behind the baptistry wall.

  - Press the “Spa Off” button on the Water Heater Control Panel.  This will ensure the Water Heater Control Panel is in a reset state.

  - Press the “Spa On” button on the Water Heater Control Panel.

  - You should hear the heater blower turn on. Repeat steps 1-2 above if you do not hear anything.

  - Adjust the temperature using the up and down arrow buttons.  An optimal water temperature for a baptism is between 83 – 90° F, warmer for hotter days and cooler for colder days.

# Two Hours Prior to Baptism:

- Turn the Water Pump and Waterfall On utilizing the On Demand Timer Switch.  Set the time as needed to have the baptism.

- Turn the Water Heater located behind the baptistry wall to “Spa On” if it is not already turned on.

# Just Prior to Baptism:

- Note:  It doesn’t matter whether the waterfall is running during a baptism.  If you do not want the waterfall to run during the baptism turn the pump/waterfall off by setting the On Demand Timer Switch to “Off” and setting the Baptistry Timer to “System Off”.

# Following the Baptism:

- Set the Baptistry Timer to “Run”.

- Set the On Demand Timer Switch to “Off”.

- Turn the Water Heater Control Panel to “Heater Off”.

On Demand Timer Switch

Water Heater Control Panel

Water Heater

Water Heater Control Panel

Cover Plate

Baptistry Timer